Національний технічний університет України

«Київський політехнічний інститут імені Ігоря Сікорського»

Факультет інформатики та обчислювальної техніки

Кафедра інформаційних систем та технологій

# Лабораторна робота №6

з дисципліни «Інженерія систем ІоТ»

Тема: «Використання реле в IoT застосунках»

Варіант 106

| Виконав: студент групи ІА-34 Тунік Олександр | Перевірив: асистент кафедри ІСТ Головатенко І. А. |
| --- | --- |

Мета: Навчитися використовувати реле у IoT застосунках, та зрозуміти доцільність його використанян у тих чи інших випадках.

# Хід роботи

## Крок 1: Додавання і базове програмування датчика вологості ґрунту

Ініціалізуємо counterfit застосунок як ми робили раніше. Додаємо сенсор вологості грунту на порт 106:

Задамо діапазон рандомних показань згідно з варіантом.

Зчитувати значення з датчика будемо через АЦП (from counterfit_shims_grove.adc import ADC).

## Крок 2: Додавання реле до CounterFit

В тому самому застосунку додаємо реле:

Для програмування імпортуємо (from counterfit_shims_grove.grove_relay import GroveRelay)

Програмуємо базовий функціонал реле.

## Крок 3: Управління з допомогою MQTT

Додаємо MQTT функціонал до проекту з сенсором вологи грунту. Створюємо MQTT сервер. Створюємо базовий функціонал серверу. Все як в минулих лабораторних.

Сенсор надсилає дані:

## Крок 4: Додаємо синхронізацію на сервер

На сервер додамо логіку для увімкнення реле на 5 секунд, потім відписування від телеметрії на 20 секунд.

Функціонал сервера:

## Код сенсора:

import time import json from counterfit_shims_grove.adc import ADC from counterfit_connection import CounterFitConnection from counterfit_shims_grove.grove_relay import GroveRelay import paho.mqtt.client as mqtt CounterFitConnection.init('127.0.0.1', 5000) adc = ADC() relay = GroveRelay(107) id = '8d15a2d7-258e-4955-824f-875c02b45292' client_name = id + 'soil_moisture_sensor_client' client_telemetry_topic = id + '/telemetry' server_command_topic = id + '/commands' mqtt_client = mqtt.Client(mqtt.CallbackAPIVersion.VERSION2, client_name) mqtt_client.connect('test.mosquitto.org') mqtt_client.loop_start() print("MQTT connected!") def handle_command(client, userdata, message): payload = json.loads(message.payload.decode()) print("Message received:", payload) if payload['relay_on']: relay.on() else: relay.off() mqtt_client.subscribe(server_command_topic) mqtt_client.on_message = handle_command while True: try: soil_moisture = adc.read(106) except Exception as e: print(f"Error reading sensor: {e}") telemetry = json.dumps({'soil_moisture': soil_moisture}) print("Sending telemetry:", telemetry) mqtt_client.publish(client_telemetry_topic, telemetry) time.sleep(10)

## Код сервера:

import json import time import threading import paho.mqtt.client as mqtt id = '8d15a2d7-258e-4955-824f-875c02b45292' client_telemetry_topic = id + '/telemetry' client_name = id + 'soil_moisture_sensor_server' server_command_topic = id + '/commands' mqtt_client = mqtt.Client(mqtt.CallbackAPIVersion.VERSION2, client_name) mqtt_client.connect('test.mosquitto.org') mqtt_client.loop_start() water_time = 5 wait_time = 20 def send_relay_command(client, state): command = {'relay_on': state} print("Sending message:", command) client.publish(server_command_topic, json.dumps(command)) def control_relay(client): print("Unsubscribing from telemetry") mqtt_client.unsubscribe(client_telemetry_topic) send_relay_command(client, True) time.sleep(water_time) send_relay_command(client, False) time.sleep(wait_time) print("Subscribing to telemetry") mqtt_client.subscribe(client_telemetry_topic) def handle_telemetry(client, userdata, message): payload = json.loads(message.payload.decode()) print("Message received:", payload) if payload['soil_moisture'] > 881: threading.Thread(target=control_relay, args=(client,)).start() mqtt_client.subscribe(client_telemetry_topic) mqtt_client.on_message = handle_telemetry while True: time.sleep(2)

Лабораторна робота успішно виконана.

Репозиторій з виконанням можна переглянути за: https://github.com/Smegalex/Engineering_IoT_systems